# Open Source SPM Controller Software

This documentation summarizes the implemented functions, and where to find the related modules and more detailed documentation. Real time controller runs on an FPGA target. User interface is handled on a computer that runs the ‘Host’ code. Following diagram illustrates the modular structure of the software. Implemented main functions on the target are shown in green boxes. Number of the loops that act for the given function is written for each box. On the brown boxes you can find the information received from or sent to the host computer by the target. Each box has the name of the front panel VIs that are linked to given function.

The code on the software has a PI block to control the Z output. One channel from the sampled analog input channels is selected and then the process variable that PI controller runs on is extracted. Process variable calculation is where the user implements imaging modes like contact, amplitude modulation, off-resonance tapping. The process variable of the selected imaging mode block is fed to PI controller and that generates the Z output.

The other important block is the ‘Scan Engine’. For the given scanning parameters and coarse scanning waveforms it interpolates the X and Y scanner signals, counts down for pixel, line, and frame data flags.

With the information from Scan Engine and Z Control block, image data is sorted and averaged into pixel data and sent to the Host.

# Target

For the detailed target documentation please follow the ‘Target’. Following you can find the name of the loops in the vi runs on the FPGA target.

- Populate memory items for feedback from host

- AIO settings

- Analog inputs acquisition

- Analog inputs sorting

- x-y physical output

- Slow host-controlled signal generator

- Height and excitation output

- Fast ADC DAC interface

- Input module acquisition and excitation output

- ORT Mode

- AM mode

- Contact mode average

- ORT mode excitation generator

- Main process variable selection

- PID calculation and height control

- Retract on background correct to get non-interact background

- z Watchdog for auto-approach

- Coarse Z control

- Host data transfer interaction wave generator and memory control

- Transfer interaction data to host

- When all requested elements sent, raise interrupt

- Transfer background to host

- Host data averaging

- Host data transfer main

- Aux inputs recording and passover to averaging

- Aux inputs averaging and pixel synchronous output

- Aux bundle to host

- Memory write access for scan engine

- scan loop

- Automatic background correct request

- Triggers End of Line / Frame

- Trigger pixel

- Write a Sawtooth Waveform to 3 FIFOs (Stick-Slip)

- SPI Comm Loop

# Host

For the detailed host documentation please follow the ‘User Manuel’. Following you can find the list of front panel VIs and how to locate them in Menu.vi

- Menu

  - File

    - Abort

  - Graphs

    - Oscilloscope

    - Scope

      - Scoping analog inputs and outputs of the FPGA in low speed (1KSPS)

    - Monitor

      - Image display monitor

    - 2xMonitor

    - 3xMonitor

    - 4xMonitor

    - X-Y Waveforms

      - Coarse scan waveform generation

  - System

    - Load Configuration

      - Cantilever parameters and calibration

      - Piezo calibration

      - Image channels

      - Vertical deflection correction

    - Analogs

      - Gain, offset, filters, inversion settings on analog input and outputs

    - Signals

  - Main

    - Z controller

      - Imaging mode selection

      - Coarse Z stage control

      - Z piezo control

      - Feedback control

    - Scan

      - Scan control

      - Image saving

    - Excitation

      - Excitation generation in ORT and AM mode

      - Background correction and display in ORT mode

    - Ramp

      - Ramp on Z signal to calculate the deflection sensitivity of the cantilever

    - Photodiode

  - Supplementary

    - Thermal Tune Calibration

      - Thermal tune and cantilever parameters fit

    - LA Controller

      - High order controller on Z signal

    - ClosedLoopScan

      - Closed loop control on X and Y signals

    - XYZStage

    - Bias

    - Analog Outputs

    - SCIM

  - External

    - Excitation ZI

    - InterferometerConfig